TECHNICAL SERVICE REPORT

Water Analysis

| Customer: | Location: | Date: | PO#: |
| --- | --- | --- | --- |
| Chick-fil-A | #2521 | 9/25/09 |  |
| Address: |  | Tested By: |  |
| River Crossing FSU 600 Corwin Nixon Blvd. South Lebanon, OH |  | AR |  |
| Phone: | Email: | Selecto Contact: |  |

| Recommendation: |  |
| --- | --- |

##### ANALYTICAL RESULTS

| Evaluation of: | Target Water Sample | Starbuck’s Water Sample | Recommended Max Levels |
| --- | --- | --- | --- |
| Fluoride, ppm | 2.6 | 3.0 | 2 ppm |
| Chloride, ppm | 30 | 32 | 250 ppm |
| Nitrite, ppm | ND | ND | 1 ppm as N |
| Nitrate, ppm | 3.4 | 3.4 | 10 ppm as N |
| Sulfate, ppm | 99 | 89 | 250 ppm |
| Sodium, ppm | 182 | 194 | 250 ppm |
| Chloramine | ND | ND | .5 ppm |
| Chlorine, ppm | ND | ND | Detectable |
| Alkalinity, ppm | 310 | 320 | 150 ppm |
| Total Dissolved Solids, ppm | 500 | 600 | 500 ppm |
| Hardness, ppm | 10 | 8 | 100 ppm |
| pH | 7.3 | 7.4 | 6.5 – 8.5 ppm |
| Turbidity | 0.00 | 0.00 | .05 NTU |

The information supplied is believed to be accurate and is only indicative of the water tested.  The testing laboratory is not state certified.

3980 Lakefield Court    Suwanee, GA 30024    678.475.0799    800.635.4017

www.selectoinc.com

sales@selectoinc.com